Product Technical Datasheet

Version: V1.0

Revision Date: 2025-06-10

Basic Information

- Chemical Name：Methyl 3-Methoxypropionate

- Abbreviation：MMP

- CAS No.：3852-09-3

- Molecula Formula: C5H10O3

- Molecular Weight：118.13g/mol

- Appearance：Colorless and transparent liquid with no visible impurities.

- Odor：It has a slight ether-like odor.

Physicochemical Indicators

| Item | Industrial Grade Standard |
| --- | --- |
| Purity (GC) | ≥99.0% |
| Acid Value (mgKOH/g) | ≤0.20 |
| Moisture Content (%) | ≤0.10 |
| Color (Pt-Co) | ≤15 Hazen |
| Density (20℃, g/mL) | 1.010±0.002 |
| Flash Point (closed cup, ℃) | ≥47 |
| Boiling Point (at Atmospheric Pressure, ℃) | 142-148 |
| Refractive Index (n₂₀^D) | 1.4005-1.4040 |

- Main Application Fields

- Coatings and Inks Industry

Automotive Coatings: Used in original equipment manufacturer (OEM) coatings and refinish coatings to improve the leveling property and glossiness of the coating film, and solve problems such as sagging and dark bubbles;

Wood Coatings: Prevents aluminum powder from coarsening and darkening in high-gloss clear varnishes, and improves the dispersibility of aluminum silver paste;

Inks: Serves as a solvent for screen printing inks to enhance adhesion to plastic and metal substrates.

- Electronic Chemicals Field:

Photoresist Strippers: Efficiently dissolve photoresist residues and cause no corrosion to silicon wafers or glass;

Electronic Cleaning Agents: Replace diethylene glycol dimethyl ether (diglyme) to improve cleaning efficiency and safety.

- Pharmaceuticals and Fine Chemicals:

Pharmaceutical Intermediates: Used in the synthesis of drugs such as tyrosine kinase inhibitors;

Pesticide Formulations: Act as a solvent to enhance the permeability and stability of active ingredients.

- Other Fields

Alternative to PMA: Its solvency power is superior to propylene glycol methyl ether acetate (PMA), with a faster volatilization rate, which can reduce the use of defoamers;

Adhesives: Adjusts the curing speed and bonding strength of epoxy adhesives and hot-melt adhesives.

Packaging Specifications

Industrial grade: 180kg iron drum

Precautions for Storage (and Use)

It should be stored in a cool, dry and well-ventilated place, away from ignition sources, heat sources and oxidizing agents; direct sunlight should be prevented, and the container should be kept sealed; the storage area should be equipped with leakage emergency treatment equipment and appropriate containment materials.

Shelf Life

From the date of production, when stored under specified storage conditions, the product can be kept sealed for 24 months. It needs to be re-inspected after expiration.

Precautions

When operating, wear goggles and chemical-resistant gloves to avoid direct skin contact.

The indicators and data provided in this manual are based on our current technical knowledge and practical experience. They are for reference only and do not constitute any guarantee. In view of the limitations of this product and the diversity of user requirements, please strictly test the adaptability of this product and the stability of the final product during use to verify whether it is suitable for your intended processes and applications and can achieve the predetermined objectives. You must also strictly comply with local safety, environmental protection regulations and operating specifications. The compiler of this manual shall not be liable for any losses caused by improper use or storage. The Technical Support and Customer Service Center of our company's Marketing Department is willing to provide you with consultation and application technical services related to the product. You are welcome to contact us by letter or phone.

Contact Address: No. 12 Taihe North Road, B1 District, Zhaoqing,China

Customer Service Center Hotline: +86(758)-2698552         Fax: +86(758)-2699551

http://www.zhidianchem.com                          E-mail：msd@zhidianchem.com